Chinhoyi University of Technology

SCHOOL OF ENGINEERING SCIENCES AND TECHNOLOGY

DEPARTMENT OF ICT AND ELECTRONICS
(COMPUTER ENGINEERING)

Project Proposal

Name: Takudzwa Kelvin Mukaro

Reg Number: C21145793S

Level: 5.2

# 1. Title

Predictive Maintenance System for Solar Mini-Grids using Machine Learning

# 2. Background

Zimbabwe’s demand for power has outgrown the supply and is resulting in loadshedding. This shortage has forced households, farmers, and Small-to-Medium Enterprises (SMEs) to invest in solar photovoltaic (PV) systems. Solar energy offers a clean and decentralized alternative, but the systems themselves are very complex, including sensitive components such as PV panels, inverters, and battery storage banks.

Regardless of the high initial investment required for these systems, most mini-grids lack automated monitoring and maintenance mechanisms. Users only rely on manual inspections which are infrequent, prone to human error, and unable to detect internal component degradation until it is too late. Scheduled visual inspections mostly miss common issues like deterioration of the battery, inverter overloads sometimes, partial shading of panels, and abnormal temperatures.

When these faults happen the system can fail completely, and the consequences are very severe, from unexpected downtime to expensive emergency replacements and safety risks such as battery fire. Adding to that, public trust in solar systems is lost. It is now possible to shift from reactive maintenance to predictive maintenance by using low-cost IoT sensors and AI, specifically ML. Intelligent systems can now identify possible failures by evaluating real-time operational data thus allowing cost-effective actions that ensure supply continuity.

# 3. Problem Statement

Since the development of solar mini-grids, they have suffered worse than expected performances and sudden malfunctions due to lack of active monitoring and predictive maintenance systems. The use of scheduled manual and reactive maintenance causes serious problems, such as:

- Unexpected Downtime: Systems can fail without giving a warning thus disrupting critical activities for users.

- Reduced Component Lifespan: Abnormal charging/discharging of solar batteries and overheating in inverters can degrade these components quickly if not corrected immediately.

- High Operational Costs: Preventative maintenance is much more cheaper than emergency repairs or to some extent component replacements.

- Lack of Visibility: In areas like rural areas, the users lack technical expertise near them to even identify root causes of underperformance.

This research aims to cover this gap by implementing a low-cost machine learning predictive maintenance system since there is currently no affordable, automated and intelligent tool locally that can address these failures and give alerts in real-time.

# 4. Objectives

## 4.1 Main Objective

To design and implement a predictive maintenance system using Machine Learning and IoT sensors that is able to monitor, detect and predict faults in solar mini-grids.

## 4.2 Specific Objectives

## The study aimed at a number of objectives to achieve our main objective and below are the specific objectives:

- To simulate and generate an operational dataset for solar mini-grids that includes PV output, inverter load, battery voltage, and temperature parameters so that we establish baseline behavior.

- To do feature engineering on these parameters and also store in the same dataset.

- To collect real-time measurements from system components using IoT data collection module with ESP32 microcontrollers.

- To train Random Forest model for battery health prediction.

- To train Isolation Forest model for anomaly detection.

- To develop a nodejs backend system that handles operations and a React dashboard with real-time monitoring, visualization, and can raise alert notifications.

- To study accuracy metrics, the prediction error and the system's responsiveness under changing working conditions so that we can evaluate the system's performance.

# 5. Significance of the Study

This system is of great importance to a lot of stakeholders in the energy sector due to significant operational, economic and policy-related benefits if it is to be implemented successfully such as:

- Operational Efficiency: The system reduces downtime and ensures reliable supply of power.

- Cost Reduction: Being alert for future faults help save costs for users because they will have planned maintenance.

- Extended Component Life: Continuous monitoring of components extends lifespan, for example, the continuous monitoring of battery health prevents abusive usage patterns like deep cycling.

- Contribution to Research: This study can provide help and insights to the body of knowledge on smart energy management and the application of AI in renewable energy within the Zimbabwean context and predictive maintenance systems in general.

- Scalability: This system is built in modules, thus making it scalable and adaptable.

# 6. Limitations / Delimitations

### Limitations

- Hardware Availability: Low cost sensor alternatives were considered for the prototype due to budget and availability constraints.

- Environmental Variations: While the simulation accounted for real-world weather conditions like cloud cover and ambient heat to be varying unpredictably, the prototype testing was laboratory controlled.

- Training Data: The lack of years of historical data for local mini-grids limited the ability to train for long-term seasonal degradation patterns since Machine Learning models depend on large volumes of data.

### Delimitations / Mitigation Measures

- Data Simulation: Long-term historical data was not collected due to time constraints of the project, and instead, simulated and synthetic datasets were used to train machine learning models and model system behavior.

- Prototype Hardware: Instead of a high-voltage industrial deployment our prototype was          limited to the laboratory so as to reduce risks and costs and just prove the proof of concept.

# 7. Brief Literature Review

Recent studies indicate that predictive maintenance has become an essential component in renewable energy system optimization (Antonio Fresco Navacerrada, 2024). Machine Learning techniques such as Random Forest, Support Vector Machines, and Autoencoders have shown effectiveness in anomaly detection and component health estimation.
(https://www.researchgate.net/publication/385382721)

Solar PV systems are susceptible to various faults, including module degradation, partial shading, hot spots, maximum power point tracking (MPPT) failures, and inverter malfunctions (SmartHelio in article, Faults, January 2025), to mention a few. Early detection of these faults is essential for maintaining optimal system performance. Mellit and Pavan proposed an artificial neural network (ANN) based monitoring system for fault detection in grid-connected PV plants. Their approach achieved 95.2% accuracy in detecting module failures but was limited to specific fault types. Chouder and Silvestre developed a fault detection method based on power loss analysis that achieved 89.6% accuracy in identifying performance degradation, though it required precise system modeling. Dhimish et al introduced a statistical approach using t-test calculations for detecting multiple fault types with an accuracy of 93.7%. With the advent of IoT technologies, several researchers have proposed integrated monitoring solutions. Garoudja et al presented an IoT framework utilizing wireless sensor networks and support vector machines (SVM) for fault classification, achieving 92.1% accuracy. Harb et al developed a low-cost IoT monitoring system using Arduino microcontrollers and cloud storage, but their solution lacked advanced analytics capabilities.
(World Journal of Advanced Research and Reviews, 2022, 14(03), 857-870 859)

Researchers have applied ML to wind turbines, grid inverters, and battery lifespan prediction. However, application in solar mini-grids, particularly in sub-Saharan Africa, remains limited. Existing commercial systems are expensive and require technical expertise, making them unsuitable for rural deployments.

The major research gap lies in the development of affordable, real-time, and locally-applicable predictive maintenance systems for solar mini-grids. This project addresses this by integrating low-cost IoT sensors with ML models and a user-friendly dashboard.

# 8. Research Methodology

### 8.1 System Architecture

This system will consist of:

- IoT Layer: ESP32 microcontroller + PZEM/SCT sensors measuring voltage, current, load, and temperature.

- Backend Layer: Node.js API + Python ML engine.

- Machine Learning Models:

  - Random Forest Regression for battery health prediction

  - Isolation Forest/Autoencoder for fault detection

- Frontend Layer: React dashboard for real-time monitoring

- Database: MySQL

### 8.2 Data Collection

- Simulated data for 30 days using Python.

- Real-time hardware sensor data (voltage, current, temperature).

### 8.3 Data Processing

- Cleaning and normalization

- Feature extraction

- Labelling normal vs faulty patterns

- Feature Engineering

### 8.4 Machine Learning Development

- Train models using 70/30 split

- Evaluate using RMSE, accuracy, and anomaly scores

- Deploy the model via REST API

### 8.5 Prototype Development

- Build full system pipeline: sensor → backend → ML → dashboard → alerts

### 8.6 Evaluation

- Test accuracy of predictions

- Measure system response time

- Conduct reliability tests on hardware

9 Gantt Chart

References

Akyaz, T., & Engın, D. (2024). Machine learning-based predictive maintenance system for artificial yarn machines. IEEE Access, 12, 125446-125461. https://doi.org/10.1109/ACCESS.2024.3454548

Mellit, A., Pavan, A.M., Kalogirou, S.A. (2011). Application of Artificial Neural Networks for the Prediction of a 20-kWp Grid-Connected Photovoltaic Plant Power Output. In: Gopalakrishnan, K., Khaitan, S.K., Kalogirou, S. (eds) Soft Computing in Green and Renewable Energy Systems. Studies in Fuzziness and Soft Computing, vol 269. Springer, Berlin, Heidelberg. https://doi.org/10.1007/978-3-642-22176-7_10

A. Chouder and S. Silvestre, "Power loss analysis in photovoltaic systems through a monitoring approach," Sol. Energy, vol. 84, no. 3, pp. 326-336, 2010.

M. Dhimish, V. Holmes, B. Mehrdadi, M. Dales, and P. Mather, "Multiple fault detection and identification algorithm for photovoltaic systems," IEEE Trans. Device Mater. Reliab., vol. 18, no. 3, pp. 448-456, 2018.

E. Garoudja, F. Harrou, Y. Sun, K. Kara, A. Chouder, and S. Silvestre, "Statistical fault detection in photovoltaic systems using IoT-enabled monitoring," Sol. Energy, vol. 155, pp. 1090-1101, 2017.

K. A. Harb, A. Nuhic, M. C. E. Tano, and C. Batunlu, "Internet of Things based monitoring system for photovoltaic arrays," Renew. Energy, vol. 80, pp. 777-788, 2015.