Chinhoyi University of Technology

SCHOOL OF ENGINEERING SCIENCES AND TECHNOLOGY

DEPARTMENT OF ICT AND ELECTRONICS
(COMPUTER ENGINEERING)

Project Proposal

Name: Takudzwa Kelvin Mukaro

Reg Number: C21145793S

Level: 5.2

# 1. Title

Machine Learning Based Predictive Maintenance System for Solar Mini-Grids

# 2. Background

Due to frequent grid outages, rising electricity costs, and the need for rural electrification Zimbabwe and many developing countries are increasingly adopting solar mini-grid systems as an alternative energy source. However, most of these mini-grids lack automated monitoring and maintenance systems. Faults such as battery deterioration, inverter overload, abnormal temperature rises, PV(Photovoltaic) underperformance to mention only a few, often go unnoticed until failure occurs. Failures lead to high downtime, expensive component replacement, safety risks, and reduced trust in solar technologies but with advancements in IoT and Machine Learning (ML), it is now possible to implement predictive maintenance systems that monitor component behavior in real time and forecast some failures before they occur. This project aims to use IoT devices and ML algorithms to assess solar mini-grid performance and provide early fault detection and predictive insights.

# 3. Problem Statement

Solar mini-grids experience performance issues and unexpected failures due to lack of active monitoring and predictive maintenance. As of now, most systems rely on manual inspection, which is not often, reactive, costly, and often inaccurate. This leads to:

- Unexpected breakdowns and system downtime

- Shortened lifespan of batteries and inverters

- High maintenance costs due to late fault detection

- Challenges in identifying root causes of failures

- Limited technical expertise in remote areas

There is no affordable, automated, intelligent tool that predicts failures and alerts users early.
This project aims to address this big gap by designing a machine learning-powered predictive maintenance system for solar mini-grids.

# 4. Objectives

## 4.1 Main Objective

To design and implement a machine learning based predictive maintenance system capable of monitoring, detecting, and predicting faults in solar mini-grid components.

## 4.2 Specific Objectives

- To simulate and generate operational datasets for solar mini-grids, including PV output, inverter load, battery voltage, and temperature parameters.

- To design and implement an IoT-based data acquisition module for collecting real-time measurements from solar mini-grid components.

- To develop and train machine learning models for fault detection, anomaly identification, and battery health prediction using the collected data.

- To design and implement a backend system and React dashboard for real-time monitoring, visualization, and alert notifications.

- To evaluate the system’s performance using accuracy metrics, prediction error, system responsiveness, and reliability under varying operating conditions

# 5. Significance of the Study

- Reduces system downtime by predicting faults before they occur.

- Extends the lifespan of batteries and inverters through early warnings.

- Supports rural electrification and sustainability goals.

- Saves costs for households, farmers, SMEs, and institutions relying on mini-grids.

- Contributes to smart energy management and renewable energy research in Zimbabwe.

- Provides a scalable framework that can be used in other energy systems.

# 6. Limitations / Delimitations

### Limitations

- Availability of physical solar mini-grid hardware for data collection.

- Limited historical datasets for training ML models.

- Variations in weather and environmental conditions may affect data consistency.

### Delimitations / Mitigation Measures

- Use simulated and synthetic datasets to model system behavior during early stages.

- Incorporate ESP32/PZEM sensors for real-time experimental data.

- Design system in a modular manner to adapt to different mini-grid configurations.

# 7. Brief Literature Review

Recent studies indicate that predictive maintenance has become an essential component in renewable energy system optimization. Machine Learning techniques such as Random Forest, Support Vector Machines, and Autoencoders have shown effectiveness in anomaly detection and component health estimation.
(https://www.researchgate.net/publication/385382721)

Solar PV systems are susceptible to various faults, including module degradation, partial shading, hot spots, maximum power point tracking (MPPT) failures, and inverter malfunctions. Early detection of these faults is essential for maintaining optimal system performance. Mellit and Pavan [8] proposed an artificial neural network (ANN) based monitoring system for fault detection in gridconnected PV plants. Their approach achieved 95.2% accuracy in detecting module failures but was limited to specific fault types. Chouder and Silvestre [9] developed a fault detection method based on power loss analysis that achieved 89.6% accuracy in identifying performance degradation, though it required precise system modeling. Dhimish et al. [10] introduced a statistical approach using t-test calculations for detecting multiple fault types with an accuracy of 93.7%. With the advent of IoT technologies, several researchers have proposed integrated monitoring solutions. Garoudja et al. [11] presented an IoT framework utilizing wireless sensor networks and support vector machines (SVM) for fault classification, achieving 92.1% accuracy. Harb et al. [12] developed a low-cost IoT monitoring system using Arduino microcontrollers and cloud storage, but their solution lacked advanced analytics capabilities.
(World Journal of Advanced Research and Reviews, 2022, 14(03), 857-870 859)

Researchers have applied ML to wind turbines, grid inverters, and battery lifespan prediction. However, application in solar mini-grids, particularly in sub-Saharan Africa, remains limited. Existing commercial systems are expensive and require technical expertise, making them unsuitable for rural deployments.

The major research gap lies in affordable, real-time, and locally-adaptable predictive maintenance systems for solar mini-grids. This project addresses this by integrating low-cost IoT sensors with ML models and a user-friendly dashboard.

# 8. Research Methodology

### 8.1 System Architecture

This system will consist of:

- IoT Layer: ESP32 microcontroller + PZEM/SCT sensors measuring voltage, current, load, and temperature.

- Backend Layer: Node.js API + Python ML engine.

- Machine Learning Models:

  - Random Forest Regression for battery health prediction

  - Isolation Forest/Autoencoder for fault detection

- Frontend Layer: React dashboard for real-time monitoring

- Database: MySQL

### 8.2 Data Collection

- Simulated data for 30 days using Python.

- Real-time hardware sensor data (voltage, current, temperature).

### 8.3 Data Processing

- Cleaning and normalization

- Feature extraction-

- Labelling normal vs faulty patterns

### 8.4 Machine Learning Development

- Train models using 70/30 split

- Evaluate using RMSE, accuracy, and anomaly scores

- Deploy the model via REST API

### 8.5 Prototype Development

- Build full system pipeline: sensor → backend → ML → dashboard → alerts

### 8.6 Evaluation

- Test accuracy of predictions

- Measure system response time

- Conduct reliability tests on hardware